# Changing Default Settings

## Change the Default Units

| Navigate to Spotter Server Settings |  |
| --- | --- |
| Go to Settings Tab |  |
| Set Speed to Meters per Second and Distance to Meters |  |

# Creating Filters

Create filters by going to filter and clicking the + button

## People Filter

| Filter: People is a default, so you just need to modify the default People filter. - Remove the upper bounds of speed - Change duration to 5 seconds - Select appropriate radars (usually can set to all ground radars) - Replace distance with Displacement greater than or equal to 10m |  |
| --- | --- |

## Unknown Filter

| Filter: Unknown is not a default and needs to be created manually - Set type = to Unknown with 0% confidence - Select appropriate radars (usually can set to all ground radars) |  |
| --- | --- |

## Vehicle Filter

| Filter: Vehicles is a default, so you just need to modify the default Vehicles filter. - Remove the lower bounds of speed - Change duration to 5 seconds - Select appropriate radars (usually can set to all ground radars) - Replace distance with Displacement greater than or equal to 15m |  |
| --- | --- |

# Creating Classifications

| Classifications are a type of Action |  |
| --- | --- |
| Create a new action by clicking the plus icon Set the type to “Classification” |  |

## Probable People Classification

| - Set the name to “Probable Person” - Set the “Type” to Person - Set the “Intent” to Unknown |  |
| --- | --- |

## Probable Vehicle Classification

| - Set the name to “Probable Vehicle” - Set the “Type” to Vehicle - Set the “Intent” to Unknown |  |
| --- | --- |

# Setting Radar Sensitivity

## Default

| - Go to settings - Go to Devices - Select desired radar - Go to “Sensitivity” |  |
| --- | --- |
| - Select the preset “Medium Wind” - In some firmware, it’s just called “Medium” |  |

# Adding Zone Classifications

## Classification: Probable People

| - Go to Settings - Go to Zones - Select desired Zone - Go to “Actions” |  |
| --- | --- |
| - Add a new action |  |
| - Set type to “classification” - Set Action to your “Classification: Probable Person” you created - Set Filter Group to People - Click “Add” |  |

## Classification: Probable Vehicle

| - Go to Settings - Go to Zones - Select desired Zone - Go to “Actions” |  |
| --- | --- |
| - Add a new action |  |
| - Set type to “classification” - Set Action to your “Classification: Probable Vehicle” you created - Set Filter Group to Vehicles - Click “Add” |  |